// BlackRoad Universal Blade Sled (OpenSCAD) // This generates a 3D printable "drawer" for your Monolith. // It is parametric - change the dimensions to fit a Luckfox, ESP32, or Milk-V.

// --- Dimensions of your Chip (Default: Luckfox Pico) --- chip_width = 21; chip_length = 51; chip_height = 10; // Clearance for components

// --- Blade Settings --- wall_thickness = 2; blade_length = 80; // Total depth of the Monolith handle_size = 15;

module blade() { difference() { // Main Body union() { // The Sled Floor cube([chip_width + (wall_thickness*2), blade_length, 2]);

// The Side Walls
        translate([0, 0, 0])
            cube([wall_thickness, blade_length, chip_height]);
        translate([chip_width + wall_thickness, 0, 0])
            cube([wall_thickness, blade_length, chip_height]);
            
        // The Handle (Front) with Grip
        translate([0, -handle_size, 0]) {
            cube([chip_width + (wall_thickness*2), handle_size, chip_height]);
            // Grip texture
            for(i=[0:3:handle_size-2]) {
                translate([0, i, chip_height])
                    cube([chip_width + (wall_thickness*2), 1, 1]);
            }
        }
    }
    
    // --- Cutouts ---
    
    // Airflow Holes (Hex pattern usually best, using slots for simplicity)
    for(i=[10:10:blade_length-10]) {
        translate([wall_thickness + 2, i, -1])
            cube([chip_width - 4, 5, 4]);
    }
    
    // Power Wire Channels (Back)
    translate([wall_thickness + 2, blade_length - 5, -1])
        cube([3, 10, 5]); // Positive
    translate([chip_width + wall_thickness - 5, blade_length - 5, -1])
        cube([3, 10, 5]); // Ground
        
    // Status LED Light Pipe Hole (Front Handle)
    translate([(chip_width/2)+wall_thickness - 1.5, -handle_size-1, chip_height/2])
        cube([3, handle_size+2, 3]);
}

}

// Render blade();

// --- Instructions --- // 1. Measure your chip (Milk-V, Luckfox, etc). // 2. Adjust 'chip_width' and 'chip_length' variables above. // 3. Render (F6) and Export as STL. // 4. Print in PLA/PETG. // 5. Slide chip in (use double-sided tape or hot glue for "No Solder" fit).